CHAPTER FOUR
IMPLEMENTATION AND DOCUMENTATION

# 4.1 Introduction

This chapter presents the implemented battery monitoring software prototype, its measured outputs, and verification results. The work uses the Panasonic 18650PF 25 °C drive-cycle data to estimate an approximate state-of-charge (SoC) proxy from voltage, current, battery-case temperature, and elapsed time. It reports model comparison, resource characterization, C export and host-side simulation, followed by system testing and operational guidance. The reported target is a capacity-referenced coulomb-counting proxy, not independently verified physical SoC. No physical sensor, microcontroller, or electrical power test was performed.

# 4.2 Implementation

The development used Python 3.10.20 with NumPy, pandas, SciPy and scikit-learn for data processing and model experiments, and a C99 compiler for deployment-format inference. The data archive contains one Panasonic 18650PF cell. Ten separate 25 °C drive-cycle runs were prepared at approximately 1 Hz, producing 112,701 rows from 1,126,083 source observations. The proxy was calculated as 100 × [1 + (Ah at sample − Ah at run start) / 2.77493 Ah]. The reference capacity is the mean of two measured 25 °C start-of-tests discharge throughputs (2.79826 Ah and 2.75160 Ah); the measured Ah channel was not supplied as a model input. The public dataset record is identified by DOI 10.17632/wykht8y7tg.1 (Mendeley Data, n.d.).

Input validation checks finite values, required fields, within-cycle time order, and duplicate cycle/time keys. Model comparisons use a fixed complete-cycle split: seven runs (75,375 samples) for training and three runs (37,326 samples) for testing. The feature scaler is fit on training data only. The selected Random Forest uses 100 trees, at most 128 leaf nodes per tree, and a minimum leaf size of two. A 50-tree Random Forest was retained as a smaller candidate. Both forests were exported as standalone C99 headers and run through a host-compiled CSV simulator.

Figure 4.1. Implementation sequence and data flow

| Stage / actor | Operation and output |
| --- | --- |
| Measured archive → preparation | Load the ten separate 25 °C drive-cycle files; validate channels and timestamps; create the capacity-referenced proxy and 1 Hz feature table. |
| Preparation → model evaluation | Use voltage, current, battery temperature and elapsed time; split by complete cycle; fit preprocessing on training rows; compare regressors on held-out cycles. |
| Model evaluation → C export | Save selected and reduced Random Forest artifacts; translate tree parameters to C99 headers; compile and compare outputs against Python. |
| User → C simulator → user | Provide four comma-separated features per row; simulator returns indexed proxy predictions, or reports an error for invalid input/model selection. |

# 4.3 Result

Results below are reported as observed in the saved experiment outputs. Prediction metrics describe agreement with the derived proxy only. They do not measure error against validated physical SoC.

## 4.3.1 Testcase 1: Comparison of regression models

The three regressors were evaluated on the same 37,326 held-out samples from Cycle 2, Cycle 4, and LA92. Lower MAE and RMSE indicate closer agreement with the proxy; higher R² indicates more variation in the proxy was explained on this split.

Table 4.1. Fixed complete-cycle holdout comparison

| Model | MAE (percentage points) | RMSE (percentage points) | R² |
| --- | --- | --- | --- |
| Ridge Regression | 3.1264 | 5.0750 | 0.9700 |
| Random Forest (100 trees) | 2.2584 | 3.0700 | 0.9890 |
| Extra Trees | 3.2223 | 4.3091 | 0.9784 |

The Random Forest recorded the lowest aggregate MAE and RMSE of the three candidates on this split and was retained as the provisional selected baseline.

## 4.3.2 Testcase 2: Reduced Random Forest and host resource results

The 50-tree candidate was compared with the selected 100-tree model using the same cycles. Measurements below are host-computer software results; they are not microcontroller measurements.

Table 4.2. Accuracy and resource comparison

| Measure | 100-tree selected model | 50-tree candidate |
| --- | --- | --- |
| Held-out proxy MAE | 2.2584 percentage points | 2.2950 percentage points |
| Joblib model size | 1,868,529 bytes | 934,929 bytes |
| C header size | 1,210,081 bytes | 606,478 bytes |
| Median latency, batch of 32 | 38.69 ms per batch | 24.91 ms per batch |
| Median latency, batch of 256 | 47.72 ms per batch | 25.34 ms per batch |

The 50-tree artifact was about 50.0% smaller on disk than the selected 100-tree artifact. Its measured MAE was higher by about 0.0366 percentage points on the shared holdout. Batch timing was measured on an Intel Core i5-4210U Linux host, with model.predict timed after warm-up. Process RSS includes Python, scientific libraries, input data and runtime overhead; it is not a model-only memory estimate.

## 4.3.3 Testcase 3: C99 export and simulator output

The compiled C implementation was compared with the corresponding Python estimators for every held-out row. Each model was checked on 37,326 samples using an absolute tolerance of 1 × 10⁻⁹. Maximum observed absolute difference was 4.2633 × 10⁻¹⁴ proxy percentage points for both models, and both validations passed.

A two-row example was then passed to the host simulator. The input fields are voltage (V), current (A), battery temperature (°C), and elapsed time (s), in that order.

Table 4.3. Sample simulator test data and output

| Input features (V, A, °C, s) | Selected model proxy (%) | 50-tree candidate proxy (%) |
| --- | --- | --- |
| 3.9, -0.5, 25.3, 120 | 98.603052 | 98.611326 |
| 3.7, -1.1, 26.1, 900 | 78.999203 | 79.195152 |

Simulator output includes sample index, model label, and soc_proxy_percent. These example predictions demonstrate execution and formatting, not calibrated battery readings.

# 4.4 Discussion of Result

On the fixed split, Random Forest tracked the generated proxy more closely than Ridge Regression and Extra Trees under the selected aggregate metrics. The 50-tree version substantially reduced stored artifact size while changing MAE only slightly on this comparison. This is a useful size–proxy-agreement trade-off for continued software evaluation, but does not establish that the smaller model is superior in an embedded system.

The close C/Python agreement shows that the exported forest logic preserved the tested estimator outputs. It verifies a software translation over the tested held-out feature rows; it does not verify sensor acquisition, state initialization, charge integration, compiler behavior on a microcontroller, or electrical power demand. Host-side latency and memory figures are specific to the recorded computer and runtime.

Interpretation is constrained by the target itself. Each run is assumed to begin near full charge and shares an early measured capacity reference; capacity may change between runs. The 25 °C experiments cover one physical cell. The fixed holdout was used during model selection and optimization, so these scores are not an independent final estimate. Proxy values outside 0–100% were retained rather than clipped, but that does not make them physically validated. Independent SoC reference measurements, multiple cells, temperatures, and a fresh evaluation split are needed before making accuracy or generalization claims.

# 4.5 System Testing

Focused automated tests covered model export, simulator behavior, and benchmark reporting. The export and simulator tests compile the generated C path where a C compiler is available; the simulator tests also exercise invalid records and unknown model selection.

Table 4.4. System test summary

| Test case | Test activity | Observed result |
| --- | --- | --- |
| Python-to-C export | Compile test/export harness; compare forest outputs and check generated header artifacts. | Passed; held-out export parity also passed for 37,326 rows per model within 1 × 10⁻⁹. |
| CSV simulator | Run selected and candidate modes; test valid rows, wrong-width and non-finite rows, and unknown model. | Passed; valid predictions returned and invalid requests failed explicitly. |
| Resource benchmark | Check deterministic batch construction, latency summaries, invalid durations, and JSON/CSV report fields. | Passed; 10 tests and 4 subtests passed across the three focused test modules. |

The test run used tests/test_panasonic_c_export.py, tests/test_panasonic_edge_simulator.py, and tests/test_panasonic_resource_benchmark.py. It completed with 10 passed and 4 subtests passed. These results verify the tested software path, not the physical battery monitoring system.

# 4.6 Conversion

No operational legacy system was replaced, and the prototype has not been deployed to users or physical hardware. A direct production conversion is therefore not claimed. The appropriate future approach is a controlled pilot followed by phase-in: first reproduce the offline results from the documented environment, then compare the prototype in parallel with an independently validated SoC reference, and only then consider a hardware pilot. A hardware release should be blocked until sensor calibration, startup SoC initialization, safety bounds, target-device resource use, power consumption and failure handling are independently verified. The current Python workflow and saved artifacts remain the research reference during any future pilot.

# 4.7 Documentation

Operational instructions for the current host simulator are as follows:

1. Activate the project Conda environment from the repository: conda activate ml_env.

2. Create the build directory and compile the C99 simulator: mkdir -p build && cc -std=c99 -O2 -Wall -Wextra -Werror edge/simulation/panasonic_edge_simulator.c -lm -o build/panasonic_edge_simulator.

3. Supply one CSV row per sample with exactly four finite numeric fields in the order voltage (V), current (A), battery temperature (°C), elapsed time (s).

4. Select the baseline with --model selected (default) or the reduced candidate with --model candidate. The output fields are sample_index, model, soc_proxy_percent.

5. Treat each returned value as an approximate proxy prediction. Do not interpret it as a physical SoC reading or use it for safety-critical control.

Malformed, incomplete, or non-finite input rows and unknown model choices are rejected with an error. The simulator is a host-side command-line demonstration; it does not contain drivers, serial-port handling, model initialization from sensors, a graphical interface, or firmware installation. The source dataset and derived data are local project inputs and are not distributed in the simulator package.

# 4.8 System Maintenance

Maintenance of this research prototype should preserve reproducibility and the evidence boundary. Keep raw data separate from processed outputs; retain model and export provenance; record software/compiler versions; and rerun data checks, focused tests and C/Python parity after code, model, or dependency changes. Regenerate model artifacts and reports only from the documented scripts and reviewed inputs. Before comparing a replacement model, use a new cycle- or cell-level evaluation set not used for selection. Review the target-capacity assumptions and dataset license whenever inputs change. Any future hardware-specific maintenance must add sensor calibration, firmware versioning, safe error states and measured device power rather than inferring power from software size or host timings.

## Reference cited in this chapter

Mendeley Data. (n.d.). Panasonic 18650PF Li-ion Battery Data [Data set]. https://doi.org/10.17632/wykht8y7tg.1